# 🚀 Smart Airport Rover: Master Development Roadmap

## Phase 1: The Communications Core (Audio & NLP Brain) ( Completed )

Objective: Build an isolated, highly accurate voice command center that listens efficiently, understands intent despite mispronunciations, and speaks naturally.

### 1.1 Custom Wake Word Engine (COMPLETED)

- Built a dynamic audio bridge using the Google Speech API.

- Set custom wake phrase to "System Online" and added rate-limit error handling.

- Enterprise Standby: Codebase is modularized and ready to instantly swap to Picovoice (pvporcupine) for deployment on dedicated hardware.

### 1.2 Speech-to-Text Upgrade (COMPLETED)

- Keep the microphone hot to listen only after the wake word is triggered.

- Send this secondary audio clip to Google STT to capture the full user command (e.g., "turn on the camera", "move to gate 3").

### 1.3 NLP Intent Engine (Fuzzy Matching) (COMPLETED)

- Implement rapidfuzz or spaCy.

- Map user variations (e.g., "go to plane", "head to aircraft") to a single Maps_Aircraft intent.

- Add auto-correction for minor mic misinterpretations.

### 1.4 High-Fidelity Voice Output (COMPLETED)

- Replace pyttsx3 with edge-tts.

- Configure a professional, low-latency voice profile.

### 1.5 Phase 1 Testing (COMPLETED)

- Run an isolated script to ensure the robot wakes up, understands a fuzzy command, and replies naturally.

### Phase 2: The Vision System (Standard Deep Learning Integration) ( Completed )

Objective: Deploy a real-time object detection pipeline using pre-trained neural networks to identify airport hazards and personnel.

2.1 AI Framework Deployment (COMPLETED)

- Initialize the YOLOv8 Nano (yolov8n.pt) deep learning architecture for high-speed, zero-shot inference.

- Optimize the model to run efficiently on local CPU hardware without requiring dedicated cloud GPUs.

2.2 Dynamic Class Filtering (COMPLETED)

- Configure the AI matrix to ignore irrelevant data (e.g., chairs, animals) to save processing power.

- Isolate and track only security-critical COCO dataset classes: Persons, Luggage (Suitcases/Backpacks), and Vehicles (Airplanes/Trucks).

2.3 Threat Detection Logic (COMPLETED)

- Extract raw bounding box coordinates (X, Y, Width, Height) from the AI's predictions.

- (Pending) Implement proximity logic to trigger system alerts if an object breaches a specific distance threshold.

2.4 System Validation (COMPLETED)

- Verify that the AI maintains stable framerates while processing the simulated video feeds and live camera inputs.

## Phase 3: The Simulation & Dynamic UI (The Interface) (Ongoing)

Objective: Transform the visual output into a professional, presentation-ready Heads-Up Display (HUD).

### 3.1 Top-Down Mini-Map (Radar)

- Create a 2D radar in the corner of the screen.

- Plot the Rover (Green dot), the Target (Blue dot), and real-time Obstacles (Red dots).

### 3.2 Dynamic Steering Graphics

- Build a visual trajectory line or digital steering wheel that smoothly points left/right based on obstacle calculations.

### 3.3 Telemetry Dashboard

- Overlay active NLP Intents.

- Display system health (FPS, simulated battery, exact GPS coordinates).

### 3.4 Smooth Navigation Math

- Upgrade the movement logic so the rover drives in smooth arcs rather than snapping side-to-side.

## Phase 4: Security & System Integration

Objective: Stitch the modules together using professional coding standards and add final security locks.

### 4.1 Voice Authentication Security

- Build a security lock function (e.g., "System, authenticate PIN Alpha-7").

- Ensure the rover cannot move or open cargo until unlocked.

### 4.2 Object-Oriented Refactoring

- Rewrite messy global variables into clean Python Classes (class RoverVision:, class VoiceEngine:, class RoverNavigation:).

### 4.3 The Great Merge

- Combine Phase 1, Phase 2, and Phase 3 into the final smart_rover_master.py script.

### 4.4 Final Stress Test & Debugging

- Run the full simulation to ensure multithreading works without crashing the audio or freezing the video.